| NPSTC Channel Naming Plan- Short Name Supplemental Addendum |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FCC-Designated Interoperability Channels With Short Names Included |  |  |  |  |  |  |  |  |  |  |  |  |  |
| May 6, 2008 - Subscriber Programming Shown |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Yellow = Narrow only with original FCC Docket name shown |  |  |  | Channeling: N = 11.25 kHz modulation bandwidth (narrowband) W = 16 or 20 kHz modulation bandwidth, depending upon band (wideband) |  |  |  |  |  |  |  |  |  |
| Before Rebanding/Narrowbanding (Legacy Names) |  |  |  |  |  |  | Post-Rebanding/Narrowbanding |  |  |  |  |  |  |
| Current Name | Rx FREQ | Rx CTCSS | Tx FREQ | Tx CTCSS | Channeling |  | NPSTC Name | Short Name (6 char) | Rx FREQ | Rx CTCSS | Tx FREQ | Tx CTCSS | Channeling |
| VHF LOW BAND |  |  |  |  |  |  | VHF LOW BAND |  |  |  |  |  |  |
| Locally determined | 39.4600 | 156.7 | 45.8600 | 156.7 | W |  | LLAW1 | LLAW1 | 39.4600 | 156.7 | 45.8600 | 156.7 | W |
| Locally determined | 39.4600 | 156.7 | Simplex | 156.7 | W |  | LLAW1D | LLAW1D | 39.4600 | 156.7 | Simplex | 156.7 | W |
| Pending FCC | 39.4800 is used today by numerous local agencies |  |  |  |  |  | LFIRE2 (pend) | LFIRE2 | 39.4800 | 156.7 | Simplex | 156.7 | W |
| Locally determined | 45.8600 | 156.7 | Simplex | 156.7 | W |  | LLAW3D | LLAW3D | 45.8600 | 156.7 | Simplex | 156.7 | W |
| Locally determined | 45.8800 | 156.7 | Simplex | 156.7 | W |  | LFIRE4 | LFIRE4 | 45.8800 | 156.7 | Simplex | 156.7 | W |
| VHF HIGH BAND |  |  |  |  |  |  | VHF HIGH BAND |  |  |  |  |  |  |
| VCALL | 155.7525 | none | Simplex | none | N |  | VCALL10 | VCAL10 | 155.7525 | none* | Simplex | 156.7 | N |
| VTAC1 | 151.1375 | none | Simplex | none | N |  | VTAC11 | VTAC11 | 151.1375 | none* | Simplex | 156.7 | N |
| VTAC2 | 154.4525 | none | Simplex | none | N |  | VTAC12 | VTAC12 | 154.4525 | none* | Simplex | 156.7 | N |
| VTAC3 | 158.7375 | none | Simplex | none | N |  | VTAC13 | VTAC13 | 158.7375 | none* | Simplex | 156.7 | N |
| VTAC4 | 159.4725 | none | Simplex | none | N |  | VTAC14 | VTAC14 | 159.4725 | none* | Simplex | 156.7 | N |
| Locally determined | 154.2800 | none | Simplex | none | W |  | VFIRE21 | VFIR21 | 154.2800 | 156.7 | Simplex | 156.7 | N |
| Locally determined | 154.2650 | none | Simplex | none | W |  | VFIRE22 | VFIR22 | 154.2650 | 156.7 | Simplex | 156.7 | N |
| Locally determined | 154.2950 | none | Simplex | none | W |  | VFIRE23 | VFIR23 | 154.2950 | 156.7 | Simplex | 156.7 | N |
|  |  |  |  |  |  |  | VFIRE24 | VFIR24 | 154.2725 | 156.7 | Simplex | 156.7 | N |
|  |  |  |  |  |  |  | VFIRE25 | VFIR25 | 154.2875 | 156.7 | Simplex | 156.7 | N |
|  |  |  |  |  |  |  | VFIRE26 | VFIR26 | 154.3025 | 156.7 | Simplex | 156.7 | N |
|  |  |  |  |  |  |  | VMED28 | VMED28 | 155.3400 | 156.7 | Simplex | 156.7 | N |
|  |  |  |  |  |  |  | VMED29 | VMED29 | 155.3475 | 156.7 | Simplex | 156.7 | N |
| NLEMARS | 155.4750 | none | Simplex | none | W |  | VLAW31 | VLAW31 | 155.4750 | 156.7 | Simplex | 156.7 | N |
|  |  |  |  |  |  |  | VLAW32 | VLAW32 | 155.4825 | 156.7 | Simplex | 156.7 | N |
| Current Name | Rx FREQ | Rx CTCSS | Tx FREQ | Tx CTCSS | Channeling |  | NPSTC Name | Short Name (6 char) | Rx FREQ | Rx CTCSS | Tx FREQ | Tx CTCSS | Channeling |
| UHF |  |  |  |  |  |  | UHF |  |  |  |  |  |  |
| UCALL | 453.2125 | none | 458.2125 | none | N |  | UCALL40 | UCAL40 | 453.2125 | none* | 458.2125 | 156.7 | N |
| UCALLD | 453.2125 | none | Simplex | none | N |  | UCALL40D | CAL40D | 453.2125 | none* | Simplex | 156.7 | N |
| UTAC1 | 453.4625 | none | 458.4625 | none | N |  | UTAC41 | UTAC41 | 453.4625 | none* | 458.4625 | 156.7 | N |
| UTAC1D | 453.4625 | none | Simplex | none | N |  | UTAC41D | TAC41D | 453.4625 | none* | Simplex | 156.7 | N |
| UTAC2 | 453.7125 | none | 458.7125 | none | N |  | UTAC42 | UTAC42 | 453.7125 | none* | 458.7125 | 156.7 | N |
| UTAC2D | 453.7125 | none | Simplex | none | N |  | UTAC42D | TAC42D | 453.7125 | none* | Simplex | 156.7 | N |
| UTAC3 | 453.8625 | none | 458.8625 | none | N |  | UTAC43 | UTAC43 | 453.8625 | none* | 458.8625 | 156.7 | N |
| UTAC3D | 453.8625 | none | Simplex | none | N |  | UTAC43D | TAC43D | 453.8625 | none* | Simplex | 156.7 | N |
| 800 MHz |  |  |  |  |  |  | 800 MHz |  |  |  |  |  |  |
| ICALL | 866.0125 | 156.7 | 821.0125 | 156.7 | W |  | 8CALL90 | CAL90 | 851.0125 | 156.7 | 806.0125 | 156.7 | W |
| ICALLD | 866.0125 | 156.7 | Simplex | 156.7 | W |  | 8CALL90D | CAL90D | 851.0125 | 156.7 | Simplex | 156.7 | W |
| ITAC 1 | 866.5125 | 156.7 | 821.5125 | 156.7 | W |  | 8TAC91 | TAC91 | 851.5125 | 156.7 | 806.5125 | 156.7 | W |
| ITAC 1D | 866.5125 | 156.7 | Simplex | 156.7 | W |  | 8TAC91D | TAC91D | 851.5125 | 156.7 | Simplex | 156.7 | W |
| ITAC 2 | 867.0125 | 156.7 | 822.0125 | 156.7 | W |  | 8TAC92 | TAC92 | 852.0125 | 156.7 | 807.0125 | 156.7 | W |
| ITAC 2D | 867.0125 | 156.7 | Simplex | 156.7 | W |  | 8TAC92D | TAC92D | 852.0125 | 156.7 | Simplex | 156.7 | W |
| ITAC 3 | 867.5125 | 156.7 | 822.5125 | 156.7 | W |  | 8TAC93 | TAC93 | 852.5125 | 156.7 | 807.5125 | 156.7 | W |
| ITAC 3D | 867.5125 | 156.7 | Simplex | 156.7 | W |  | 8TAC93D | TAC93D | 852.5125 | 156.7 | Simplex | 156.7 | W |
| ITAC 4 | 868.0125 | 156.7 | 823.0125 | 156.7 | W |  | 8TAC94 | TAC94 | 853.0125 | 156.7 | 808.0125 | 156.7 | W |
| ITAC 4D | 868.0125 | 156.7 | Simplex | 156.7 | W |  | 8TAC94D | TAC94D | 853.0125 | 156.7 | Simplex | 156.7 | W |
| NOTE: For VHF Low Band, 156.7 Hz is recommended as a national standard for emergency use. However, it is advisable to follow the national law enforcement CTCSS plan to minimize atmospheric skip interference that can plague this band during periods of high sunspot activity. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE: 6 character short name to only be used in radios that can not support the full 8-character name. If the longer NPSTC-recommended name is 6 characters or less, it is also used for the short name, otherwise the name is abbreviated. 800 MHz short names have been approved by NPSTC. Other short names will be addressed during the ANSI standardization process. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE: Tx CTCSS for paired UHF and 800 MHz channels may vary to permit transmitter steering for multi-site systems. However, use of multi-CTCSS base receivers is recommended so that systems always respond to 156.7 Hz as a national emergency tone. For 800 MHz channels, CTCSS plans and operational procedures often are addressed in 800 MHz Regional Plans. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (*) NOTE: At a future date to be determined during the ANSI standardization process, it is recommended that all nationwide interoperability channels have 156.7 Hz CTCSS on both receive and transmit frequencies. During the transition period, it is recommended that channels marked with an asterisk and already narrowbanded (VCALL/TAC and UCALL/TAC channels) should be programmed for CTCSS on subscriber transmit only, with carrier squelch on receive. |  |  |  |  |  |  |  |  |  |  |  |  |  |